| Beth Israel Deaconess Medical Center Perioperative Services Manual |  |  |
| --- | --- | --- |
| Title: | Room Turnover and Cleaning |  |
| Policy #: | PSM 200-110 |  |
| Purpose: | Environmental cleaning and sanitation of the operating room will be done to provide a clean environment for surgical patients and be carried out in a manner that minimizes staffs’ and patients’ exposure to potentially infectious microorganisms. |  |
| Policy Statement: - Environmental cleaning protocols will be implemented for all surgical procedures. - OR personnel will demonstrate competency in standard room cleaning procedures. - Personnel will wear personal protective equipment (PPE) when cleaning the ORs. Guideline(s) for Implementation: - General Guidelines - Cleaning will be performed before, during and at the completion of each surgical procedure. - Terminal cleaning in the operating room will be performed outlined in PSM 200-404 Terminal Cleaning of the OR Suites. - Before the Surgical Procedure - The Perioperative RN will Visually inspect and assess the OR equipment and room surfaces for cleanliness prior to the beginning of all surgical procedures - Any soiled equipment or room surfaces will be cleaned by designated personnel - All horizontal surfaces should be damp dusted before the first surgical procedure of the day - During the Surgical Procedure: - Confine all contaminated/soiled items used in the course of the procedure to the immediate vicinity of the surgical area. - Promptly clean areas outside the field contaminated by organic debris, such as blood, tissue or body fluids, with the hospital approved detergent-germicide and a lint free cloth. - Discard used surgical sponges into kick-buckets lined with clear plastic bags for easier visualization. At case end, place all bagged sponges into a red biohazard bag for disposal. - Use gloves when handling sponges, equipment and instruments - Place specimens in leak-proof containers for safe transport to designated destination. - Keep traffic in and out of the room to a minimum - Keep OR doors closed at all times - Place used linens in impervious white plastic linen bags. - Place non-hazardous disposable items in clear/cloudy plastic trash bags. - Place saturated disposable items that are saturated with blood/body fluids or contaminated with tissue, into leak-proof RED biohazard waste bags. - Conclusion of Surgical Procedure (also refer to PSM 200-215 Instrument Cleaning and Table Breakdown): - End of case cleanup is a joint effort done by support, nursing and anesthesia personnel. - All items that came in contact with the patient and/or the sterile field are considered contaminated and must be discarded or decontaminated. - Sort instruments and separate sharps into appropriate containers or pans as outlined in PSM 200-215 Instrument Cleaning and Table Breakdown. - Spray all instrumentation used in procedure with Enzymatic Solution prior to transfer to CPD. - Place instruments on dirty case cart (West campus) or back table (East campus). - Discard all sharps into the puncture-resistant biohazard containers. - Discard all items saturated and non-saturated as described above. - Disconnect and cap all suction containers for transport - Utilizing hospital approved detergent/germicide, clean horizontal surfaces of furniture and equipment, all surfaces of the OR bed including the underside of the mattress pads. - Clean and return all equipment to appropriate areas of storage. - The Anesthesia tech will be responsible for all used anesthesia equipment and supplies. - Spot clean walls, ceiling, and surgical lights if necessary. - Mop floor and dispose of mop-head. - Contain all surgical waste and instrument kits on a dirty case cart (West) or back table (East) for removal from the room. - Clean Exterior surfaces of blood coolers and return with all unused blood products to blood bank. - Return x-rays to the designated area in each OR. - Make up OR bed with clean linens and set up bed in appropriate position for next surgical procedure. |  |  |
| ## Vice President Sponsor: |  | ## Marsha Maurer RN,MSN |
| ## Author/Owner/Chair: |  | ## Elena Canacari, RN, CNOR ## Director, Perioperative Services |
| ## Approved By: |  |  |
| ## OR Executive Committee/Date: |  | ## James Hurst MD, Acting Chief, Department of Surgery, ## Brett Simon MD, Chair, Department of Anesthesiology and Critical Care ## Elena Canacari RN,CNOR |
| ## OR Practice Council/Date: 04/2010 |  |  |
| Original Date Approved: 3/2001 Revisions: (Dates): 05/2010,04/2009, 11/2007, 9/2003, 6/2004 Next Review Date: 05/2013 Eliminated: (Date) |  |  |

Courtesy of Beth Israel Deaconess Medical Center, Boston, MA.